Report Embedded System

Project Name

Smart Attendance Using Keypad ID.

Team Members

| NAME | ID |
| --- | --- |
| 1- Abdelrahman Mohamed Hafez | 42310129 |
| 2- Ahmed Walied Mohamed | 42310053 |
| 3- Yousef Mohamed Abdelaziz | 42310123 |
| 4- Mohamed Tamer Mohamed | 42310012 |
| 5- Mohamed Taha | 42310086 |

Supervisor

ENG:- Bakinam Mohamed

Workload Distribution

| Team Members | Contribution |
| --- | --- |
| 1- Abdelrahman Mohamed Hafez | --------- |
| 2- Ahmed Walied Mohamed | --------- |
| 3- Yousef Mohamed Abdelaziz | --------- |
| 4- Mohamed Tamer Mohamed | --------- |
| 5- Mohamed Taha | --------- |

Date

May 2026

Table of Contents:-

Abstract1

Introduction and Motivation2

Target System3

Embedded Software Architecture4

Code Structure5

Implementation Details6

System Operation Flow 7

Hardware Components 8

Results and Testing 9

Conclusion 10

Future Work 11

1.Abstract

This project presents a Smart Attendance and Door Access Control System using Arduino and multiple embedded peripherals. The system combines fingerprint authentication, password verification, ultrasonic sensing, servo motor door control, LCD display interaction, keypad input, and attendance recording through a Python application.

The main objective of the project is to provide a secure and automated attendance and access management system. Each authorized user must first verify their fingerprint and then enter a personal password to gain access. After successful authentication, the system records attendance information into a CSV file with the user ID, user name, date, time, and status.

The system also includes several security features such as lock mode after multiple wrong password attempts, buzzer alarm, administrator unlocking mode, and unauthorized access detection. The final implementation demonstrates the integration between embedded hardware and software systems to achieve a complete real-time smart security solution.

2. Introduction and Motivation

Traditional attendance systems suffer from several problems such as manual errors, fake attendance registration, low security, and inefficient data storage. In many environments such as universities, laboratories, and companies, there is a growing need for secure and automated access control systems.

The motivation behind this project is to design a smart embedded system capable of:

- Automatically recording attendance.

- Preventing unauthorized access.

- Improving security using biometric authentication.

- Reducing human interaction and manual recording.

- Integrating hardware and software into one complete system.

The project uses fingerprint authentication because fingerprints are unique for every individual and difficult to duplicate. Additionally, combining fingerprints with password verification increases the overall security level of the system.

The attendance records are automatically saved using a Python application connected to the Arduino serial port. This provides a digital database that can later be analyzed or managed easily.

3. Target System

The target system is a smart embedded access control and attendance system that performs the following tasks:

- Detects if a person is standing near the system using an ultrasonic sensor.

- Requests fingerprint authentication from the user.

- Matches the fingerprint against stored fingerprints.

- Requests password entry using a keypad.

- Verifies the password associated with the detected user.

- Opens the door using a servo motor if authentication is successful.

- Displays system messages on the LCD.

- Records attendance data into a CSV file using Python.

- Activates security mode after multiple wrong attempts.

The system is suitable for:

- University laboratories

- Offices

- Restricted rooms

- Smart attendance systems

- Embedded security applications

4. Embedded Software Architecture

The software architecture of the system is divided into several modules. Each module is responsible for a specific functionality.

4.1 Input Layer

This layer handles all input devices:

- Fingerprint sensor

- Keypad

- Ultrasonic sensor

- Reset button

- Admin button

4.2 Processing Layer

This layer contains the main decision-making logic:

- User authentication

- Password checking

- Lock system management

- Attendance tracking

- Access validation

4.3 Output Layer

This layer controls output devices:

- LCD display

- Servo motor

- LEDs

- Buzzer

4.4 Data Communication Layer:- This layer handles serial communication between Arduino and Python for attendance storage.

5. Code Structure

The Arduino code is organized into multiple sections for readability and modularity.

5.1 Libraries:-

The following libraries are used:

| Library | Function |
| --- | --- |
| Wire.h | I2C communication |
| LiquidCrystal_I2C.h | LCD control |
| Servo.h | Servo motor control |
| EEPROM.h | Memory handling |
| PCF8574.h | Keypad expansion |
| SoftwareSerial.h | Serial communication |
| Adafruit_Fingerprint.h | Fingerprint sensor control |

5.2 Main Components Initialization:-

The code initializes:

- LCD display

- PCF8574 I/O expander

- Servo motor

- Fingerprint sensor

- Ultrasonic sensor

- LEDs

- Buzzer

5.3 User Database:-

The system stores:

- User passwords

- User names

- Fingerprint IDs

Example:

String userIDs [USERS_COUNT]

String userNames [USERS_COUNT]

int fingerIDs [USERS_COUNT]

Each fingerprint is linked to a specific password and user name.

5.4 Keypad Scanning Function:-

The keypad is connected using the PCF8574 I/O expander.

The function:

char getKey()

scans rows and columns to determine which key is pressed.

5.5 Fingerprint Verification:-

The function:

int getFingerprintID()

performs:

- Fingerprint image capture.

- Image conversion.

- Fast fingerprint matching.

- Returning the matched fingerprint ID.

5.6 Attendance Management:-

The system tracks user access using:

int accessCount[USERS_COUNT]

Rules:

- First successful access = Attendance.

- Second successful access = Leave.

- More than two accesses are denied.

6.Implementation Details

6.1 Ultrasonic Sensor Operation

The ultrasonic sensor continuously measures the distance between the system and nearby objects.

If:

distance < 20

the system assumes a person is present and starts authentication.

If no person is detected:

- The session resets.

- LCD returns to “Waiting...”.

6.2 Fingerprint Authentication

The user places a finger on the sensor.

The system:

- Captures the fingerprint.

- Searches stored templates.

- Identifies the user.

If the fingerprint is not recognized:

- Access is denied.

- Buzzer alarm activates.

- LCD displays “Unauthorized”.

6.3 Password Verification

After successful fingerprint recognition:

- The user enters a 4-digit password.

- Password is compared with the stored password.

If correct:

- Green LED turns on.

- Servo motor opens the door.

- Attendance is recorded.

If incorrect:

- Red LED activates.

- Buzzer alarm sounds.

- Wrong attempt counter increases.

6.4 Security Lock Mode

If:

wrongCount >= 3

the system enters lock mode.

In lock mode:

- System access is blocked.

- Red LED remains ON.

- Buzzer alarm continuously sounds.

6.5 Admin Unlock System

An administrator can unlock the system by:

- Pressing the admin button.

- Entering the admin key.

Admin key:

String adminKey = "2005";

If the key is correct:

- System unlocks.

- Alarm stops.

- Wrong attempts reset.

6.6 Servo Motor Door Control

The servo motor controls the physical door.

Door positions:

- 0° → Closed

- 90° → Open

The motor automatically closes the door after a short delay.

6.7 Attendance Recording Using Python

A Python program receives serial data from Arduino.

Example transmitted data:

0129,Abdelrahman,Attendance

The Python application:

- Reads serial data.

- Gets current date and time.

Saves records into:

attendance.csv

CSV columns:

- ID

- Name

- Date

- Time

- Status

7. System Operation Flow

The system operates according to the following sequence:

- System waits for person detection.

- Ultrasonic sensor detects nearby user.

- LCD asks for fingerprint.

- Fingerprint verification occurs.

- User enters password.

- Password validation occurs.

- Door opens if authentication succeeds.

- Attendance data is transmitted to Python.

- Python saves data into CSV file.

- System resets and waits for the next user.

8. Hardware Components

| Component | Purpose |
| --- | --- |
| Arduino Uno | Main controller |
| Fingerprint Sensor | User authentication |
| LCD I2C Display | User interface |
| Keypad | Password input |
| PCF8574 Module | Keypad expansion |
| Ultrasonic Sensor | Person detection |
| Servo Motor | Door control |
| LEDs | Status indication |
| Buzzer | Alarm system |
| Push Buttons | Reset and admin control |

9.Results and Testing

The system was tested under multiple scenarios.

Successful Authentication

- Fingerprint recognized.

- Correct password entered.

- Door opened successfully.

- Attendance saved correctly.

Unauthorized Fingerprint

- System denied access.

- Alarm activated successfully.

Wrong Password Attempts

- After three attempts, system entered lock mode.

Admin Recovery

- Admin key successfully unlocked the system.

Attendance File

The CSV file stored all attendance records correctly with accurate timestamps.

The final implementation achieved stable operation and successful integration between all hardware and software modules.

10.Conclusion

This project successfully implemented a smart attendance and access control system using embedded systems technology. The integration of fingerprint authentication, password verification, ultrasonic sensing, and automated attendance recording created a secure and efficient system.

The project demonstrated practical applications of:

- Embedded programming

- Sensor interfacing

- Real-time processing

- Security systems

- Serial communication

- Data logging

The system improved both security and automation while providing a user-friendly interface through the LCD and keypad.

Overall, the project achieved its objectives and demonstrated the importance of embedded systems in modern smart security applications.

11.Future Work

Several improvements can be added in future versions of the project:

- Store attendance data in an online database.

- Add Wi-Fi or Bluetooth connectivity.

- Create a mobile application interface.

- Add face recognition using AI cameras.

- Increase the number of supported users.

- Encrypt attendance records for higher security.

- Use EEPROM to permanently save attendance counters.

- Add real-time monitoring dashboard.

- Integrate cloud storage systems.

- Add battery backup for power failures.

CIRCIUT DIGRAM WITHOUT - FINNGERPRINT: -